LED SPECIFICATION

LED DATASHEET

| CUSTOMER： |  |  |
| --- | --- | --- |
| SUPPLIER P/N: | LED 544XC Serial |  |
| CUSTOMER P/N: | UXC Serial |  |
| DISCRIPTION: | 5.4mm*3.8mm Ellipse LED |  |
| SAMPLE DATE: | 2009.06.22 |  |
| CUSTOMER APPROVAL & SIGNATURE: |  |  |
| □ Approved □ Rejected □ Limited Approved： __________________________ |  | Approved By： |

SUPPLIER APPROVED BY:

| Eng. Dept. | QC. Dept. | Production Dept. | Marketing |
| --- | --- | --- | --- |
| 李小松 | 张雷敏 | 杨金志 | CLOUD BAI |

1. Packing Dimensions & Spatial Distribution:

| Product Model | Emitting Color |  | LENS |  | DIE Material |  |
| --- | --- | --- | --- | --- | --- | --- |
| 544XC/543XC Serials | R/G/B/Y/C/PG/W |  | Water Clear |  | GaAlInP/GaAsP |  |
| Tolerance Grade | Dimension Tolerance (UNIT: mm Tolerance ) |  |  |  |  |  |
|  | 0.5~3 | 3~6 |  | 6~30 |  | 30~120 |
| Medium(m) | ±0.1 | ±0.2 |  | ±0.3 |  | ±0.5 |

2. Absolute Maximum Ratings: (TA=25℃)

| Parameter | Symbol | Color | Maximum Rating | Unit |
| --- | --- | --- | --- | --- |
| Peak Forward Current | IFp | All | 100 | mA |
| Forward Current | IF | All | 50 | mA |
| Reverse Voltage | VR | All | 5 | V |
| Power Dissipation | Pd | Red | 110 | mW |
|  |  | Green | 110 |  |
|  |  | Yellow | 110 |  |
|  |  | Orange | 110 |  |
|  |  | Blue | 140 |  |
|  |  | Pure Green | 140 |  |
| Operating Temperature | Topr | All | -40 ~ +85 | ℃ |
| Storage Temperature | Tstg | All | -40 ~ +85 | ℃ |
| Electrostatic Discharge | ESD | All | 1000 | V |
| Soldering Temperature | 260℃ for 5 seconds |  |  |  |

Notes:

1. Absolute maximum ratings Ta=25℃.

2. Tolerance of measurement of forward voltage±0.1V.

3. Tolerance of measurement of peak Wavelength±2.0nm.

4. Tolerance of measurement of luminous intensity±15%.

5. Tolerance of measurement of angle intensity±15%.